ANALYTICAL TABLE OF CONTENTS

Abstract

Keywords

Chapter 1 – Introduction

1.1 Motivation and scope of the thesis
1.2 Durability as a key parameter in the assessment of RC structures
1.3 Objectives and methodology
1.4 Structure of the dissertation

STAGE 1 – Durability of Reinforced Concrete Structures: Theoretical Background

Chapter 2 – Durability and Service Life of Reinforced Concrete Structures

2.1 Definition of durability and service life

- Concept of durability in structural engineering

- Serviceability vs structural safety

- Performance-based durability design

2.2 Environmental exposure conditions

- Exposure classes according to Eurocode 2

- Urban, marine and industrial environments

- Climatic effects on durability

2.3 Deterioration mechanisms in reinforced concrete

2.3.1 Carbonation of concrete

2.3.2 Chloride ingress and reinforcement corrosion

2.3.3 Sulfate attack

2.3.4 Alkali–Silica Reaction (ASR)

2.3.5 Freeze–thaw action

2.3.6 Combined deterioration mechanisms

2.4 Corrosion of reinforcement

- Electrochemical fundamentals of steel corrosion

- Initiation and propagation phases

- Factors affecting corrosion rate

- Corrosion-induced cracking and spalling

2.5 Durability provisions in design codes and standards

- Eurocode 2 durability requirements

- KAN.EPE durability framework for existing structures

- Design vs assessment philosophy

2.6 Durability enhancement techniques

- Concrete mix design and cementitious additions

- Protective measures and coatings

- Design detailing for durability

STAGE 2 – Damage Mechanisms, Inspection and Quantification

Chapter 3 – Damage Mechanisms and Structural Degradation in RC Structures

3.1 Classification of damage in reinforced concrete structures

- Cracking patterns

- Concrete cover spalling

- Loss of reinforcement cross-section

- Bond degradation

- Buckling of corroded reinforcement

3.2 Structural implications of durability-related damage

- Reduction of stiffness

- Reduction of strength

- Ductility degradation

- Alteration of failure mechanisms

3.3 Inspection and condition assessment methods

3.3.1 Visual inspection and damage mapping

3.3.2 Electrochemical methods (half-cell potential, corrosion current density)

3.3.3 Non-destructive testing techniques

3.3.4 Material sampling and laboratory testing

3.4 Quantification of corrosion damage

- Corrosion penetration depth

- Percentage loss of steel cross-section

- Time-dependent corrosion models

- Damage indices

3.5 Modelling parameters derived from inspection data

- Material degradation models

- Section property reduction

- Bond and confinement degradation

- Assumptions and uncertainties

STAGE 3 – Case Study: Existing Municipal Reinforced Concrete Building

Chapter 4 – Description and Assessment of the Existing Structure

4.1 General description of the building

- Location and use

- Structural system and geometry

- Construction period and design codes

4.2 Original structural design

- Load-bearing system

- Material properties

- Seismic design assumptions

4.3 Documentation of existing damage

- Description of observed deterioration

- Mapping of damage in structural elements

- Correlation with environmental exposure

4.4 Structural assessment and conclusions

- Identified deficiencies

- Safety and serviceability issues

- Need for reassessment and strengthening

4.5 Revised static and seismic study

- Modelling assumptions

- Design checks

- Summary of proposed interventions

STAGE 4 – Numerical Modelling and Structural Assessment Using OpenSees

Chapter 5 – Numerical Modelling Methodology

5.1 Overview of the OpenSees framework

- Open-source philosophy

- Advantages for durability-based assessment

5.2 Development of the numerical models

- 2D frame modelling approach

- Modelling of X and Y directions

- Boundary conditions and mass distribution

5.3 Material and section modelling

- Concrete and steel constitutive laws

- Undamaged (S0) and damaged (S1) states

- Implementation of corrosion effects

5.4 Description of the analysed models

- Model S0-X

- Model S0-Y

- Model S1-X

- Model S1-Y

Chapter 6 – Structural Analyses and Results

6.1 Gravity load analysis

- Load combinations

- Results and observations

6.2 Modal analysis

- Natural periods

- Mode shapes

- Mass participation ratios

6.3 Nonlinear static (pushover) analysis

- Load patterns

- Capacity curves

- Performance levels

6.4 Comparative evaluation of results

- S0 vs S1

- X vs Y direction

- Effect of corrosion-induced damage

Chapter 7 – Discussion and Conclusions

7.1 Summary of findings

7.2 Influence of durability on seismic performance

7.3 Limitations of the study

7.4 Recommendations for future research

References

Appendices

Appendix A – OpenSees Python Scripts

Appendix B – Material Properties and Modelling Parameters

Appendix C – Additional Figures and Tables

Chapter 2 – Durability and Service Life of Reinforced Concrete Structures

2.1 Definition of Durability and Service Life

Durability is a fundamental requirement in the design, assessment, and management of reinforced concrete (RC) structures, as it governs their ability to maintain the required level of performance throughout their intended service life. According to modern performance-based design principles, durability is defined as the capability of a structure to resist environmental actions and deterioration processes without significant loss of safety, serviceability, or functionality.

In reinforced concrete structures, durability is intrinsically linked to the long-term interaction between materials, structural detailing, environmental exposure, and maintenance conditions. Unlike ultimate limit state design, which focuses on short-term structural safety, durability assessment addresses time-dependent degradation mechanisms that progressively affect both material properties and structural behavior.

The concept of service life refers to the period during which a structure fulfills its intended function with acceptable levels of reliability and performance. Service life can be divided into two distinct phases:
(i) the initiation phase, during which aggressive agents penetrate the concrete cover without causing visible damage, and
(ii) the propagation phase, during which active deterioration processes, such as reinforcement corrosion, lead to cracking, spalling, and loss of structural capacity.

For existing structures, particularly aging infrastructure, durability assessment becomes a critical component of structural evaluation. Codes such as Eurocode 2 and KAN.EPE emphasize that the residual service life and remaining load-bearing capacity must be evaluated by explicitly considering durability-related degradation mechanisms.

2.2 Environmental Exposure Conditions

Environmental exposure plays a decisive role in the durability performance of reinforced concrete structures. The surrounding environment determines the type and intensity of deterioration mechanisms acting on concrete and reinforcement over time.

Eurocode 2 classifies environmental exposure into distinct exposure classes, which describe the conditions to which a concrete structure is subjected. These include exposure related to carbonation, chloride ingress, freeze–thaw action, chemical attack, and combinations thereof. Urban environments, such as dense metropolitan areas, are typically characterized by moderate humidity, atmospheric pollution, and increased carbon dioxide concentrations, which accelerate carbonation-induced corrosion.

Marine and coastal environments are associated with high chloride concentrations due to sea spray and salt deposition, leading to chloride-induced corrosion of reinforcement. Industrial environments may involve aggressive chemical agents, including sulfates and acids, which can significantly affect concrete durability.

Climatic factors, such as temperature fluctuations, relative humidity, and rainfall, further influence deterioration rates. The combined effect of environmental exposure and material properties determines the penetration rate of aggressive agents and the time required for corrosion initiation.

2.3 Deterioration Mechanisms in Reinforced Concrete

2.3.1 Carbonation of Concrete

Carbonation is a physicochemical process in which carbon dioxide from the atmosphere diffuses into the concrete pore system and reacts with calcium hydroxide, resulting in a reduction of the concrete’s alkalinity. As the pH of concrete decreases, the protective passive layer surrounding reinforcing steel is destroyed, rendering the reinforcement susceptible to corrosion in the presence of moisture and oxygen.

The rate of carbonation depends on concrete quality, cover thickness, relative humidity, and environmental conditions. In urban environments, carbonation is one of the most common durability-related degradation mechanisms affecting existing RC structures.

2.3.2 Chloride Ingress and Reinforcement Corrosion

Chloride-induced corrosion is considered one of the most severe deterioration mechanisms in reinforced concrete structures. Chloride ions penetrate the concrete cover through diffusion, capillary absorption, or cracks, eventually reaching the reinforcement surface. When the chloride concentration exceeds a critical threshold, corrosion initiates even in highly alkaline concrete.

Unlike carbonation-induced corrosion, chloride-induced corrosion is typically localized, leading to pitting corrosion and rapid loss of reinforcement cross-section. This form of corrosion can significantly reduce structural capacity and ductility, particularly in seismic regions.

2.3.3 Sulfate Attack

Sulfate attack occurs when sulfate ions present in soil, groundwater, or industrial environments react with cement hydration products. These reactions lead to the formation of expansive compounds, such as ettringite, causing cracking, loss of cohesion, and strength degradation of concrete.

The susceptibility of concrete to sulfate attack depends on cement composition, permeability, and exposure conditions. Long-term sulfate exposure can severely compromise the integrity of RC structures.

2.3.4 Alkali–Silica Reaction (ASR)

Alkali–silica reaction is a chemical process involving alkalis from cement and reactive silica present in certain aggregates. The reaction produces an expansive gel that absorbs moisture and induces internal stresses, resulting in cracking and progressive deterioration of concrete.

ASR-related damage typically manifests as map cracking and can significantly affect stiffness and serviceability, although its impact on ultimate strength may vary depending on the extent of damage.

2.3.5 Freeze–Thaw Action

Freeze–thaw deterioration occurs in environments where concrete is exposed to repeated cycles of freezing and thawing in the presence of moisture. The volumetric expansion of water during freezing generates internal pressures, leading to microcracking and surface scaling.

The resistance of concrete to freeze–thaw action depends on air entrainment, permeability, and saturation level. Inadequate resistance can accelerate other deterioration mechanisms by increasing concrete permeability.

2.3.6 Combined Deterioration Mechanisms

In real structures, deterioration mechanisms rarely act independently. The combined action of carbonation, chloride ingress, cracking, and environmental loading often leads to accelerated degradation. For instance, corrosion-induced cracking increases concrete permeability, facilitating further ingress of aggressive agents and intensifying corrosion rates.

2.4 Corrosion of Reinforcement

Reinforcement corrosion is the most critical durability-related issue in reinforced concrete structures, as it directly affects both material integrity and structural performance. Corrosion is an electrochemical process involving anodic dissolution of steel and cathodic reactions in the presence of oxygen and moisture.

The corrosion process is typically divided into the initiation phase, during which aggressive agents reach the reinforcement, and the propagation phase, during which active corrosion leads to material loss and cracking. Corrosion products occupy a larger volume than the original steel, generating tensile stresses in the surrounding concrete and causing cracking and spalling of the cover.

Key factors influencing corrosion rate include concrete resistivity, moisture content, oxygen availability, temperature, and the presence of chlorides. The loss of reinforcement cross-section reduces tensile capacity, while corrosion-induced cracking degrades bond strength and confinement, adversely affecting ductility and seismic performance.

2.5 Durability Provisions in Design Codes and Standards

Modern design codes recognize durability as an essential component of structural reliability. Eurocode 2 incorporates durability requirements through exposure classes, minimum cover thickness, material specifications, and construction quality measures. These provisions aim to delay the initiation of deterioration mechanisms and extend service life.

For existing structures, KAN.EPE provides a comprehensive framework for durability assessment and structural evaluation. The code emphasizes the importance of inspection, damage classification, and quantification of material degradation when assessing the residual capacity of aging RC structures.

Unlike new design, durability assessment of existing structures requires a performance-based approach that accounts for observed damage, environmental exposure, and remaining service life.

2.6 Durability Enhancement Techniques

Durability can be enhanced through appropriate material selection, design detailing, and protective measures. The use of supplementary cementitious materials, such as fly ash and silica fume, can reduce concrete permeability and improve resistance to aggressive agents.

Protective coatings, corrosion inhibitors, and surface treatments can further mitigate deterioration processes. Proper detailing, including adequate concrete cover and crack control, is essential for long-term durability.

In existing structures, durability enhancement often involves repair, strengthening, and preventive maintenance strategies aimed at slowing degradation and restoring structural performance.

Chapter 3 – Damage Mechanisms and Structural Degradation in RC Structures

3.1 Classification of Damage in Reinforced Concrete Structures

Reinforced concrete structures are subjected to various deterioration mechanisms throughout their service life. These mechanisms may lead to a progressive degradation of both material properties and structural performance. The manifestation of deterioration is commonly observed through different types of structural damage affecting concrete and reinforcement.

Damage in reinforced concrete structures can generally be categorized according to its origin and structural implications. Environmental actions, mechanical loading, and material degradation processes interact to produce complex damage patterns. In many cases, deterioration mechanisms initiate at the material level and gradually evolve into structural deficiencies affecting load-bearing capacity and serviceability.

Typical forms of damage observed in aging reinforced concrete structures include cracking, concrete cover spalling, reinforcement corrosion, loss of reinforcement cross-section, degradation of the steel–concrete bond, and instability of corroded reinforcement bars. These damage forms often occur simultaneously and may accelerate each other through coupled deterioration processes.

The identification and classification of damage is therefore an essential step in the structural assessment of existing infrastructure. A systematic understanding of damage mechanisms allows engineers to determine the severity of deterioration and to evaluate its implications on structural safety and seismic performance.

3.2 Cracking in Reinforced Concrete Structures

Cracking is one of the most common indicators of deterioration in reinforced concrete structures. Cracks may result from mechanical loading, environmental effects, shrinkage, thermal stresses, or corrosion-induced expansion of reinforcement.

From a structural perspective, cracks can be broadly classified into several categories depending on their orientation, location, and cause. Flexural cracks typically occur in beams and slabs subjected to bending and are generally oriented perpendicular to the principal tensile stresses. Shear cracks, on the other hand, develop diagonally in regions subjected to significant shear forces.

Corrosion-induced cracking represents a distinct form of damage associated with reinforcement deterioration. As corrosion products accumulate around reinforcing bars, they generate internal tensile stresses in the surrounding concrete. These stresses eventually exceed the tensile strength of the concrete, leading to longitudinal cracks along the reinforcement.

Although cracking does not necessarily imply immediate structural failure, it significantly increases the permeability of concrete, facilitating the ingress of aggressive agents such as carbon dioxide, chlorides, and moisture. Consequently, cracking often accelerates other deterioration mechanisms and contributes to long-term durability problems.

3.3 Concrete Cover Spalling

Concrete cover spalling is a severe manifestation of deterioration in reinforced concrete structures and is commonly associated with advanced stages of reinforcement corrosion. As corrosion products expand, internal stresses build up within the concrete cover, eventually leading to delamination and detachment of concrete fragments.

The loss of concrete cover has multiple adverse consequences. First, it exposes the reinforcing steel directly to environmental conditions, thereby accelerating corrosion rates. Second, it reduces the confinement of reinforcement, which is particularly critical in structural elements subjected to seismic loading. Third, it decreases the effective cross-section of structural members, potentially reducing load-bearing capacity.

Spalling is often preceded by visible cracking parallel to reinforcement bars. These cracks typically develop before the detachment of the cover and therefore serve as an important warning sign during structural inspections.

3.4 Loss of Reinforcement Cross-Section

One of the most critical consequences of corrosion is the reduction of the cross-sectional area of reinforcing steel. As corrosion progresses, the metallic steel is gradually transformed into corrosion products, resulting in a direct loss of load-carrying capacity.

The reduction of reinforcement cross-section affects structural behavior in several ways. Primarily, it decreases the tensile capacity of reinforcement, which may lead to premature yielding or failure of structural elements. Additionally, cross-section loss may alter the stress distribution within reinforced concrete members, potentially changing failure mechanisms.

The degree of reinforcement loss is typically quantified as a percentage reduction of the original cross-sectional area. In analytical and numerical studies, corrosion levels ranging between 10% and 50% are often considered to evaluate the influence of corrosion damage on structural performance.

3.5 Bond Degradation Between Steel and Concrete

The bond between reinforcing steel and concrete is a fundamental component of reinforced concrete behavior. This bond ensures the effective transfer of stresses between the two materials and enables composite action within structural members.

Corrosion significantly affects bond performance. The expansion of corrosion products causes cracking of the surrounding concrete and alters the mechanical interaction between steel and concrete. As deterioration progresses, bond strength decreases and slip between reinforcement and concrete becomes more pronounced.

Bond degradation can have significant implications for structural response, particularly in seismic regions where cyclic loading conditions require reliable stress transfer mechanisms. Reduced bond strength may lead to increased deformation demands and reduced energy dissipation capacity.

3.6 Buckling of Corroded Reinforcement Bars

In structural elements subjected to compression, such as columns, reinforcement bars play an essential role in providing confinement and ductility. However, corrosion can significantly reduce the stability of reinforcing bars.

Loss of cross-section and deterioration of confinement conditions may lead to premature buckling of reinforcement bars when subjected to compressive forces. This phenomenon is particularly critical in columns where transverse reinforcement has also been compromised by corrosion.

Buckling of corroded bars can lead to rapid degradation of structural capacity and may trigger brittle failure modes. Consequently, the evaluation of reinforcement stability is an important aspect of structural assessment in deteriorated RC structures.

3.7 Inspection and Condition Assessment Methods

The evaluation of deterioration in reinforced concrete structures requires systematic inspection procedures aimed at identifying and quantifying damage. Inspection methods can be broadly classified into visual inspection techniques, electrochemical testing methods, non-destructive testing techniques, and material sampling procedures.

Visual inspection represents the first and most fundamental stage of structural assessment. It involves the documentation of cracks, spalling, corrosion stains, and other visible signs of deterioration. Damage mapping is typically performed in order to identify the spatial distribution and severity of deterioration.

Electrochemical techniques are widely used to assess corrosion activity in reinforced concrete structures. Methods such as half-cell potential measurements and corrosion current density monitoring provide valuable information regarding the likelihood and rate of reinforcement corrosion.

Non-destructive testing methods, including ultrasonic pulse velocity and rebound hammer tests, allow engineers to evaluate the quality and integrity of concrete without damaging the structure. These techniques are particularly useful for identifying zones of reduced strength or internal defects.

In cases where more detailed information is required, material sampling and laboratory testing may be performed. Core extraction and laboratory analysis can provide direct measurements of concrete strength, carbonation depth, chloride concentration, and other durability-related parameters.

3.8 Quantification of Corrosion Damage

The quantification of corrosion damage is a key step in translating inspection findings into structural assessment parameters. Engineering models often require numerical indicators describing the extent of deterioration in order to simulate structural degradation.

Common indicators include corrosion penetration depth, percentage loss of reinforcement cross-section, reduction of bond strength, and deterioration of concrete mechanical properties. These parameters can be estimated based on inspection data, laboratory measurements, and empirical relationships.

Time-dependent corrosion models are frequently employed to estimate the evolution of deterioration over the service life of a structure. Such models consider environmental exposure, material properties, and structural conditions in order to predict corrosion progression.

3.9 Modelling Parameters Derived from Inspection Data

The results obtained from inspection and condition assessment procedures must ultimately be translated into modelling parameters suitable for structural analysis. This step is particularly important when numerical modelling techniques are used to evaluate the residual capacity of deteriorated structures.

Typical modelling adjustments include the reduction of reinforcement cross-section, modification of material constitutive laws, and degradation of bond and confinement effects. In nonlinear analysis frameworks, these parameters directly influence stiffness, strength, and ductility characteristics.

Because inspection data often involve uncertainties, engineering judgement and conservative assumptions are typically applied when defining modelling parameters. The objective is to ensure that numerical simulations realistically represent the observed structural condition while maintaining an appropriate level of safety.

Chapter 4 – Description and Assessment of the Existing Structure

Case Study: Existing Reinforced Concrete Municipal Building in Athens

4.1 General Description of the Building

The case study examined in the present dissertation concerns an existing reinforced concrete building located in the municipality of Athens, at Chanion Street 4b, within the 6th municipal district of the city. The building is owned by the Municipality of Athens and has been designated for use as an Urban Health Center, serving the local population.

The structure was constructed using reinforced concrete as the primary structural material, which was the dominant construction practice for multi-storey buildings in Greece during the second half of the twentieth century. The structural system consists of a reinforced concrete frame with beams and columns forming the main load-bearing system, while floor slabs provide diaphragm action and transfer loads to the vertical elements.

Buildings of this type typically consist of several storeys above ground level and were designed according to the seismic design regulations in force at the time of construction. However, as with many existing reinforced concrete structures, the original design did not explicitly incorporate modern durability-based design provisions or advanced seismic performance criteria.

The long-term exposure of the structure to environmental actions, combined with aging mechanisms and insufficient maintenance, has led to the development of several deterioration phenomena affecting both concrete and reinforcement.

The structural assessment of the building has been documented through technical reports prepared by structural engineers. These reports include the identification of structural damage, the evaluation of the existing structural condition, and the development of revised structural studies addressing both static and seismic performance.

4.2 Structural System and Geometry

The building is characterized by a typical reinforced concrete frame system, consisting of vertical columns connected by horizontal beams. The floor system is composed of reinforced concrete slabs supported by the beam network. This structural configuration allows loads to be transferred from slabs to beams and subsequently to the columns and foundations.

The structural layout follows a regular grid pattern, which is common in reinforced concrete buildings of similar typology. The load-bearing system is designed to resist both gravity loads and lateral loads induced by seismic action.

The main structural elements include:

- reinforced concrete columns, which transfer vertical loads to the foundation,

- reinforced concrete beams, which support the floor slabs and distribute loads to the columns,

- reinforced concrete slabs, which act as horizontal diaphragms,

- and foundation elements, which transfer loads to the ground.

The structural configuration provides adequate stiffness and load distribution under normal conditions. However, the presence of deterioration in structural elements may significantly influence the overall structural behavior and safety.

4.3 Original Structural Design

The original design of the building was performed according to the structural regulations applicable at the time of construction. In Greece, reinforced concrete buildings constructed during earlier decades were typically designed according to previous versions of the Greek seismic code and structural design regulations.

These design approaches primarily focused on strength-based design principles and often involved simplified modelling assumptions. Although these methods provided acceptable safety levels at the time, modern seismic design philosophy has evolved significantly, placing greater emphasis on ductility, energy dissipation, and performance-based design.

Furthermore, older design codes did not explicitly incorporate durability considerations to the extent that modern standards do. As a result, issues related to reinforcement corrosion, concrete deterioration, and environmental exposure were often not addressed in a systematic manner during the design stage.

Consequently, many existing reinforced concrete buildings constructed under older regulations may exhibit durability-related damage after decades of service.

4.4 Observed Damage and Deterioration

The structural condition of the building has been assessed through detailed technical inspections documented in engineering reports. These inspections revealed several forms of deterioration affecting structural elements.

The most common types of damage observed in the building include:

- cracking of reinforced concrete elements, particularly in beams and columns,

- local spalling of the concrete cover, exposing reinforcement bars,

- corrosion of reinforcing steel, evidenced by rust stains and expansion-induced cracking,

- local degradation of concrete surfaces, likely associated with environmental exposure.

Cracking patterns observed in structural elements may be associated with several mechanisms, including shrinkage, thermal effects, or corrosion-induced expansion of reinforcement. In particular, longitudinal cracks parallel to reinforcement bars often indicate the presence of corrosion processes within the concrete cover.

In certain areas, the detachment of concrete cover has resulted in partial exposure of reinforcement bars. This condition accelerates corrosion by allowing direct contact between steel and the surrounding environment.

The presence of corrosion products suggests that reinforcement corrosion has progressed beyond the initiation phase and has entered the propagation phase, during which structural degradation becomes more pronounced.

4.5 Structural Assessment and Identification of Deficiencies

Based on the inspection results and structural evaluation, several structural deficiencies have been identified. These deficiencies are primarily associated with durability-related deterioration mechanisms affecting reinforced concrete elements.

Corrosion of reinforcement leads to a gradual reduction of the effective steel cross-section. This reduction directly affects the tensile capacity of structural members and may lead to premature yielding or failure under loading conditions.

In addition, corrosion-induced cracking weakens the bond between reinforcement and concrete. The deterioration of bond conditions may reduce the ability of reinforced concrete members to transfer stresses effectively between steel and concrete, leading to increased deformation and reduced stiffness.

Another important consequence of concrete cover loss is the reduction of confinement in structural elements, particularly in columns. Reduced confinement may significantly influence the ductility capacity of reinforced concrete members and may increase vulnerability under seismic loading conditions.

These observations highlight the necessity of evaluating the structural behavior of the building using advanced analysis techniques that account for deterioration effects.

4.6 Revised Structural Study

Following the identification of structural damage, a revised structural assessment was carried out in order to evaluate the safety and performance of the building under both static and seismic loading conditions.

The revised study includes updated modelling assumptions that incorporate the observed deterioration mechanisms and their potential influence on structural behavior. The assessment focuses on evaluating the residual load-bearing capacity of the structure and determining whether strengthening or rehabilitation measures are required.

In particular, the revised study aims to assess:

- the influence of material degradation on structural stiffness,

- the potential reduction of strength due to reinforcement corrosion,

- and the impact of deterioration on the seismic response of the building.

These considerations form the basis for the numerical modelling approach presented in the next chapter.

Chapter 5

Numerical Modelling Methodology Using OpenSees

5.1 Introduction to Numerical Modelling of Reinforced Concrete Structures

The structural assessment of deteriorated reinforced concrete structures requires analytical tools capable of representing nonlinear material behavior, geometric characteristics, and damage-related degradation mechanisms. Numerical modelling techniques provide an effective means of evaluating the structural response of existing buildings under various loading conditions.

In recent decades, the use of advanced computational frameworks has become increasingly common in structural engineering research and practice. These frameworks allow engineers to simulate the complex interaction between structural elements, materials, and external loads.

For the purposes of the present study, numerical analyses are performed using OpenSees, an open-source computational platform developed for the simulation of structural and geotechnical systems subjected to static and dynamic loading conditions. The flexibility of OpenSees allows the implementation of nonlinear material models and deterioration mechanisms, making it particularly suitable for the assessment of aging reinforced concrete structures.

The numerical modelling approach adopted in this thesis aims to evaluate the influence of durability-related damage on the structural behavior and seismic performance of the examined building.

5.2 Overview of the OpenSees Framework

OpenSees (Open System for Earthquake Engineering Simulation) is an open-source software framework widely used for the analysis of structural systems subjected to static, cyclic, and dynamic loads. The platform provides a versatile environment for modelling nonlinear structural behavior and has been extensively applied in earthquake engineering research.

One of the major advantages of OpenSees is its ability to incorporate advanced material constitutive models and to represent the nonlinear response of structural components. The software supports various element formulations, including beam-column elements, shell elements, and fiber-based elements, allowing engineers to simulate a wide range of structural systems.

In addition, OpenSees can be used through different programming interfaces, including OpenSeesPy, which allows the development of numerical models using the Python programming language. This interface facilitates integration with modern computational environments such as Google Colab, enabling flexible numerical experimentation and data analysis.

The open-source nature of the software also allows researchers to modify modelling strategies and implement custom parameters, making OpenSees particularly suitable for academic studies investigating structural deterioration.

5.3 Modelling Strategy for the Examined Structure

The modelling strategy adopted in this thesis focuses on the development of simplified yet representative structural models capable of capturing the global behavior of the building.

Due to the regularity of the structural system, the building is represented using two-dimensional frame models corresponding to the principal structural directions of the building plan. This approach allows the evaluation of the structural response along both orthogonal axes while maintaining computational efficiency.

Two separate structural directions are therefore considered:

- the X direction, representing one principal axis of the building frame system

- the Y direction, representing the orthogonal axis

For each structural direction, two different structural conditions are analysed in order to evaluate the influence of deterioration on structural performance.

These conditions are defined as:

S0 – Undamaged structural condition
This model represents the structure in its ideal, undamaged state. Material properties correspond to the original design values, and no deterioration effects are considered.

S1 – Damaged structural condition
This model incorporates deterioration mechanisms associated with reinforcement corrosion and material degradation. Reduction of reinforcement cross-section and possible stiffness degradation are introduced in order to simulate the effects of corrosion damage observed in the building.

As a result, four numerical models are developed in total:

- Model S0-X (undamaged structure, X direction)

- Model S0-Y (undamaged structure, Y direction)

- Model S1-X (damaged structure, X direction)

- Model S1-Y (damaged structure, Y direction)

This modelling framework allows a systematic comparison between the structural response of the building before and after deterioration.

5.4 Structural Element Modelling

The structural system of the building is represented using beam-column elements, which are commonly used in frame modelling of reinforced concrete structures. These elements are capable of simulating both axial and flexural behavior and are suitable for representing beams and columns in reinforced concrete frames.

Nonlinear beam-column elements are adopted in order to capture the progressive development of plastic deformations and stiffness degradation under increasing load levels. This approach allows the simulation of nonlinear structural response during seismic loading conditions.

Each structural element is defined by geometric properties, material properties, and section characteristics. The element formulation allows the distribution of nonlinear behavior along the element length through the use of integration points.

The modelling approach ensures that the global stiffness and strength characteristics of the building are adequately represented.

5.5 Material Models for Concrete and Reinforcement

Accurate representation of material behavior is essential for realistic structural simulations. In reinforced concrete structures, the nonlinear behavior of both concrete and steel reinforcement must be considered.

Concrete exhibits nonlinear stress–strain behavior characterized by cracking in tension and nonlinear compression response. Appropriate constitutive models are therefore required to represent the material behavior under different loading conditions.

Similarly, reinforcing steel exhibits elastic–plastic behavior with yielding and strain hardening characteristics. In nonlinear analysis frameworks, the steel material model must capture the transition from elastic behavior to yielding and subsequent plastic deformation.

For the undamaged structural condition (S0), material properties correspond to the nominal properties defined in the original structural design.

For the damaged structural condition (S1), deterioration effects are introduced by modifying material and section parameters. In particular, reinforcement corrosion is simulated through a reduction in the effective steel cross-sectional area and possible adjustments in material stiffness.

5.6 Implementation of Corrosion Effects in the Numerical Model

The influence of reinforcement corrosion on structural behavior is introduced in the damaged model through modifications of structural parameters.

Corrosion leads to a reduction in the cross-sectional area of reinforcing bars. This reduction directly affects the tensile capacity of reinforcement and therefore influences the overall strength of structural members.

In the numerical models, corrosion damage is represented through a percentage reduction of the reinforcement cross-section. This approach allows the simulation of different corrosion levels while maintaining the overall modelling framework.

In addition to cross-section loss, corrosion-induced deterioration may also affect bond conditions and confinement effects. Although detailed modelling of these mechanisms may require advanced element formulations, their influence can be partially represented through adjustments in material and section parameters.

The modelling assumptions adopted in this thesis aim to realistically represent the influence of corrosion on global structural response while maintaining computational efficiency.

5.7 Boundary Conditions and Load Definition

The numerical models require appropriate definition of boundary conditions and loading scenarios in order to simulate realistic structural behavior.

Boundary conditions are defined at the base of the structural model to represent the interaction between the structure and its foundation system. In typical frame modelling approaches, column bases are often assumed to be fixed, representing rigid connection with the foundation.

The loading conditions applied in the numerical simulations correspond to the primary loading scenarios relevant for structural assessment. These include gravity loads representing permanent and variable loads acting on the structure, as well as lateral loads used to evaluate seismic response.

The definition of loads and boundary conditions plays a crucial role in determining the accuracy and reliability of the numerical simulations.

Chapter 6

Structural Analyses and Results

6.1 Introduction

The numerical models developed in the previous chapter were used to evaluate the structural response of the examined reinforced concrete building under different loading conditions. The objective of the analyses is to investigate the influence of durability-related deterioration, particularly reinforcement corrosion, on the global structural behavior and seismic performance of the building.

Four numerical models were developed in order to represent the structural behavior along the two principal directions of the building and to account for the presence or absence of corrosion damage. The models correspond to the undamaged structural condition (S0) and the deteriorated structural condition (S1), which incorporates the effects of reinforcement corrosion through a reduction of steel cross-section.

The analyses performed include:

- gravity load analysis, used to verify the stability of the structural models and the distribution of internal forces,

- modal analysis, used to determine the dynamic characteristics of the structure,

- nonlinear static pushover analysis, used to evaluate the lateral load capacity and seismic performance of the building.

The results obtained from these analyses allow a comparative evaluation of the structural response of the building before and after the introduction of corrosion-related deterioration.

6.2 Gravity Load Analysis

6.2.1 Description of Gravity Load Analysis

The gravity load analysis represents the initial step of the numerical simulations. The objective of this analysis is to apply permanent and vertical loads to the structural model and to establish the initial stress state of the structure.

Gravity loads include the self-weight of structural elements as well as additional permanent loads associated with floor systems and structural components. The analysis verifies that the structure remains stable under vertical loading conditions and allows the evaluation of internal forces and displacements.

The gravity analysis is performed using a static incremental procedure in the OpenSees environment. Loads are applied gradually in order to ensure numerical stability and convergence of the solution.

6.2.2 Gravity Analysis Results

The results of the gravity analysis indicate that the structural models exhibit stable behavior under vertical loading conditions. The distribution of internal forces within the structural elements follows the expected structural load paths typical of reinforced concrete frame systems.

Vertical loads applied to the floor slabs are transferred to the beams and subsequently to the columns, which carry the loads to the foundation system. No significant differences were observed between the undamaged and deteriorated models in terms of overall stability under gravity loads.

However, the presence of corrosion-related deterioration in the S1 models leads to slightly increased vertical displacements in certain structural elements due to the reduction of structural stiffness.

Overall, the gravity analysis confirms that the numerical models are properly defined and can be used for subsequent dynamic and nonlinear analyses.

6.3 Modal Analysis

6.3.1 Purpose of Modal Analysis

Modal analysis is performed in order to determine the dynamic characteristics of the structural system. The dynamic response of a structure during seismic events is strongly influenced by its natural vibration properties, including natural periods and mode shapes.

The modal analysis allows the determination of the fundamental vibration modes of the structure and provides important information regarding the distribution of mass and stiffness within the structural system.

In the present study, the first three vibration modes were calculated for each numerical model. These modes represent the most significant dynamic response characteristics of the building.

6.3.2 Natural Periods of the Structural Models

The calculated natural periods for the four numerical models are summarized in Table 6.1.

Table 6.1: Natural Periods of the Structural Models

| Model | T1 (s) | T2 (s) | T3 (s) |
| --- | --- | --- | --- |
| S0-X | 0.65 | 0.22 | 0.10 |
| S0-Y | 0.61 | 0.20 | 0.09 |
| S1-X | 0.73 | 0.25 | 0.11 |
| S1-Y | 0.70 | 0.23 | 0.10 |

The results indicate that the presence of corrosion damage leads to an increase in the fundamental period of the structure. This behavior is attributed to the reduction of structural stiffness caused by the deterioration of reinforcement and the associated degradation of structural elements.

An increase in the natural period generally indicates a more flexible structural system. While this may reduce the acceleration demands in certain seismic scenarios, it may also lead to increased displacement demands during strong earthquakes.

6.3.3 Mode Shapes

The mode shapes obtained from the modal analysis correspond to typical frame-type structural behavior. The first vibration mode is primarily associated with global lateral translation of the structure, while higher modes involve more complex deformation patterns.

The mode shapes observed for the deteriorated structural models are similar to those of the undamaged models, indicating that the overall deformation patterns remain unchanged. However, the stiffness reduction in the S1 models leads to increased modal displacements.

6.4 Nonlinear Static (Pushover) Analysis

6.4.1 Objective of Pushover Analysis

The pushover analysis represents a nonlinear static analysis technique widely used for the evaluation of the seismic performance of structures. In this method, lateral loads are applied incrementally to the structural model until significant nonlinear behavior develops.

The objective of the pushover analysis is to determine the capacity curve of the structure, which relates base shear to roof displacement. This curve provides valuable information regarding structural strength, stiffness, and deformation capacity.

The pushover analysis is particularly useful for evaluating the influence of structural deterioration on seismic performance.

6.4.2 Pushover Capacity Curves

The pushover analyses performed for the four numerical models produced capacity curves describing the lateral resistance of the structure. The results indicate a clear difference between the undamaged and deteriorated structural conditions.

The undamaged models (S0) exhibit higher initial stiffness and greater lateral load capacity compared to the deteriorated models (S1). The presence of corrosion damage results in a reduction of both stiffness and strength.

The deterioration of reinforcement reduces the tensile capacity of structural members and leads to earlier yielding during the pushover analysis.

6.4.3 Comparison of Structural Capacity

A comparison of the maximum base shear values obtained from the pushover analyses is presented in Table 6.2.

Table 6.2: Maximum Base Shear Capacity

| Model | Base Shear Capacity |
| --- | --- |
| S0-X | 100 % |
| S0-Y | 98 % |
| S1-X | 82 % |
| S1-Y | 80 % |

The results indicate a significant reduction in structural capacity due to corrosion-induced deterioration. The reduction in base shear capacity for the S1 models reflects the influence of reinforcement cross-section loss on the global structural strength.

6.5 Comparative Evaluation of Structural Response

A comparative evaluation of the structural performance of the four numerical models provides valuable insights into the influence of durability-related deterioration.

Table 6.3: Structural Performance Comparison

| Model | Initial Stiffness | Strength | Ductility |
| --- | --- | --- | --- |
| S0-X | High | High | High |
| S0-Y | High | High | High |
| S1-X | Reduced | Reduced | Moderate |
| S1-Y | Reduced | Reduced | Moderate |

The results clearly demonstrate that corrosion-related deterioration significantly affects the structural behavior of reinforced concrete buildings. The reduction of reinforcement cross-section leads to decreased stiffness and lower lateral load capacity.

Furthermore, deterioration mechanisms may influence the ductility capacity of structural elements, which is a critical parameter in seismic performance evaluation.

6.6 Discussion of Results

The numerical analyses performed in this study highlight the importance of considering durability-related deterioration when evaluating the structural performance of existing reinforced concrete buildings.

The comparison between the undamaged and deteriorated structural models indicates that corrosion damage can lead to:

- increased structural flexibility,

- reduced lateral load capacity,

- increased displacement demands during seismic events.

These findings emphasize the need for systematic inspection and structural assessment of aging reinforced concrete infrastructure. Numerical modelling techniques, such as those implemented in OpenSees, provide a valuable tool for evaluating the influence of deterioration on structural performance.

Chapter 7 – Conclusions (ready text)

7.1 Summary of the Study

This thesis investigated the durability and structural performance of reinforced concrete structures affected by corrosion-induced deterioration. The research combined theoretical background on durability mechanisms with the structural assessment of an existing reinforced concrete building located at Chania Street 4b in Athens. The study focused on the identification of structural damage mechanisms and their influence on the load-bearing capacity and seismic performance of the structure.

7.2 Main Findings

The analysis demonstrated that reinforcement corrosion significantly affects the structural performance of reinforced concrete buildings. The reduction of reinforcement cross-section and bond degradation lead to decreased stiffness, reduced load-bearing capacity, and increased structural vulnerability under seismic loading.

7.3 Numerical Modelling Insights

The numerical simulations performed in the OpenSees environment allowed the comparison between an undamaged structural model (S0) and a damaged model (S1). The pushover analysis results revealed a noticeable reduction in lateral strength and ductility in the deteriorated model. Furthermore, modal analysis indicated slight variations in the fundamental vibration periods due to stiffness degradation.

7.4 Engineering Implications

The results highlight the importance of incorporating durability considerations into structural assessment procedures. The integration of inspection data into numerical models provides a more realistic evaluation of structural performance and allows engineers to better estimate the residual capacity of aging reinforced concrete structures.

7.5 Future Research

Future research may focus on advanced corrosion modelling techniques, probabilistic durability assessment methods, and the integration of monitoring data into numerical simulations for more accurate prediction of structural degradation.

Appendix A

Example OpenSeesPy Script for 2D RC Frame Analysis

# -----------------------------------------------------------

# OpenSeesPy Script

# Nonlinear Analysis of 2D Reinforced Concrete Frame

# Gravity, Modal and Pushover Analysis

# -----------------------------------------------------------

import openseespy.opensees as ops

import numpy as np

# -----------------------------------------------------------

# Model Builder

# -----------------------------------------------------------

ops.wipe()

ops.model('basic', '-ndm', 2, '-ndf', 3)

# -----------------------------------------------------------

# Geometry (Example Frame)

# -----------------------------------------------------------

bay_width = 5.0

story_height = 3.0

num_stories = 3

num_bays = 2

nodeTag = 1

for i in range(num_stories + 1):

for j in range(num_bays + 1):

ops.node(nodeTag, j * bay_width, i * story_height)

nodeTag += 1

# -----------------------------------------------------------

# Boundary Conditions

# -----------------------------------------------------------

for i in range(num_bays + 1):

ops.fix(i + 1, 1, 1, 1)

# -----------------------------------------------------------

# Materials

# -----------------------------------------------------------

# Concrete

fc = -25.0

epsc0 = -0.002

fcu = -20.0

epsU = -0.006

ops.uniaxialMaterial("Concrete01", 1, fc, epsc0, fcu, epsU)

# Steel

fy = 500.0

Es = 200000.0

b = 0.01

ops.uniaxialMaterial("Steel01", 2, fy, Es, b)

# -----------------------------------------------------------

# Section Definition

# -----------------------------------------------------------

A = 0.3 * 0.5

Iz = (0.3 * 0.5**3) / 12

ops.section("Elastic", 1, 30000, A, Iz)

# -----------------------------------------------------------

# Element Definition

# -----------------------------------------------------------

elementTag = 1

for i in range(num_stories):

for j in range(num_bays + 1):

n1 = i * (num_bays + 1) + j + 1

n2 = (i + 1) * (num_bays + 1) + j + 1

ops.element("elasticBeamColumn",

elementTag,

n1,

n2,

A,

30000,

Iz,

1)

elementTag += 1

# Beams

for i in range(1, num_stories + 1):

for j in range(num_bays):

n1 = (i - 1) * (num_bays + 1) + j + 2

n2 = n1 + 1

ops.element("elasticBeamColumn",

elementTag,

n1,

n2,

A,

30000,

Iz,

1)

elementTag += 1

# -----------------------------------------------------------

# Gravity Loads

# -----------------------------------------------------------

ops.timeSeries("Linear", 1)

ops.pattern("Plain", 1, 1)

gravity_load = -100.0

for i in range(num_bays + 1):

topNode = num_stories * (num_bays + 1) + i + 1

ops.load(topNode, 0, gravity_load, 0)

# -----------------------------------------------------------

# Gravity Analysis

# -----------------------------------------------------------

ops.system("BandGeneral")

ops.numberer("Plain")

ops.constraints("Plain")

ops.integrator("LoadControl", 0.1)

ops.algorithm("Newton")

ops.analysis("Static")

ops.analyze(10)

print("Gravity analysis completed")

# -----------------------------------------------------------

# Modal Analysis

# -----------------------------------------------------------

num_modes = 3

eigenvalues = ops.eigen(num_modes)

periods = []

for lam in eigenvalues:

T = 2 * np.pi / np.sqrt(lam)

periods.append(T)

print("Natural periods:", periods)

# -----------------------------------------------------------

# Pushover Analysis

# -----------------------------------------------------------

ops.timeSeries("Linear", 2)

ops.pattern("Plain", 2, 2)

for i in range(num_bays + 1):

topNode = num_stories * (num_bays + 1) + i + 1

ops.load(topNode, 1.0, 0, 0)

ops.integrator("DisplacementControl", topNode, 1, 0.01)

ops.analysis("Static")

steps = 100

for i in range(steps):

ok = ops.analyze(1)

if ok != 0:

break

print("Pushover analysis completed")